RPi → AWS IoT Pipeline

Complete Setup & Configuration Guide

# 1. Project Overview

This project streams sensor / device data from a Raspberry Pi to a second server on your local network. If the second server goes offline, the data is automatically saved to AWS IoT Core and a local CSV file as a fallback.

Data flow:

RpiSend.py  →  main_receiver.py  →  second_server.py

↘  (fallback)  AWS IoT + CSV

# 2. Project Folder Structure

After cloning the GitHub repo, your folder should look like this:

rpi-iot-project/

├── config.py             ← All settings in ONE place (IPs, ports, paths)

├── main_receiver.py      ← Run on Raspberry Pi – main server script

├── RpiSend.py            ← Run on sender device – sends data

├── second_server.py      ← Run on second server – receives forwarded data

├── certs/                ← Put your AWS certificates here

│   ├── AmazonRootCA1.pem

│   ├── certificate.pem.crt

│   └── private.pem.key

├── .gitignore            ← Keeps certs/ and CSV out of GitHub

├── requirements.txt      ← Python dependencies

└── README.md             ← Quick-start reference

# 3. What You Need to Change

Open config.py — this is the ONLY file you need to edit. Everything else reads from it automatically.

## 3.1  AWS Credentials & Endpoint

| Setting | How to find / change |
| --- | --- |
| AWS_HOST | Go to AWS Console → IoT Core → Settings → copy your Endpoint URL |
| AWS_TOPIC | Change "test/topic" to your desired MQTT topic name |
| ROOT_CA_PATH | Place AmazonRootCA1.pem inside the certs/ folder — path auto-resolves |
| CERTIFICATE_PATH | Place certificate.pem.crt inside the certs/ folder — path auto-resolves |
| PRIVATE_KEY_PATH | Place private.pem.key inside the certs/ folder — path auto-resolves |

## 3.2  Network IP Addresses

| Setting | What to enter |
| --- | --- |
| RPI_HOST | IP address of the Raspberry Pi — run: hostname -I on the Pi |
| RPI_PORT | Default 12345 — only change if that port is already in use on your network |
| TARGET_HOST | IP address of your second / receiving server machine |
| TARGET_PORT | Default 5000 — only change if port is unavailable |

# 4. AWS Certificate Setup

Your certificates are already downloaded (shown in the screenshot). Follow these steps to place them correctly:

### Step 1 – Rename your files

In your downloaded folder, rename the files to match exactly:

| Your downloaded file | Rename to (place in certs/) |
| --- | --- |
| AmazonRootCA1.pem | AmazonRootCA1.pem (same) |
| certificate (Security Certificate) | certificate.pem.crt |
| private.pem.key | private.pem.key (same) |

### Step 2 – Copy into the project

On your Raspberry Pi, copy all 3 files into the certs/ folder inside the project:

cp /path/to/your/downloads/AmazonRootCA1.pem   ~/rpi-iot-project/certs/

cp /path/to/your/downloads/certificate.pem.crt ~/rpi-iot-project/certs/

cp /path/to/your/downloads/private.pem.key     ~/rpi-iot-project/certs/

The config.py automatically builds the full path — no hardcoded paths needed anymore.

# 5. GitHub Setup (First Time)

## 5.1  Create a GitHub Repository

- Go to github.com → click New repository

- Name it  rpi-iot-project  → set to Private → click Create

- Do NOT initialise with README (you will push your own)

## 5.2  Push the project from your Pi

Run these commands on the Raspberry Pi in the project folder:

cd ~/rpi-iot-project

git init

git remote add origin https://github.com/YOUR_USERNAME/rpi-iot-project.git

git add .

git commit -m "Initial commit"

git push -u origin main

Important: The .gitignore will automatically exclude the certs/ folder and any CSV files — your AWS keys will NOT be pushed.

## 5.3  Clone on a new machine later

git clone https://github.com/YOUR_USERNAME/rpi-iot-project.git

cd rpi-iot-project

pip install -r requirements.txt

Then manually copy your certs/ files (they were never in GitHub).

# 6. Installing Dependencies

## On Raspberry Pi (main_receiver.py + RpiSend.py)

pip install AWSIoTPythonSDK

## On Second Server (second_server.py)

No extra packages needed — uses Python's built-in socket library only.

# 7. Running the Project

Start the scripts in this order:

## Step 1 — Start the second server

On the machine at TARGET_HOST (192.168.2.100 by default):

python second_server.py

## Step 2 — Start the main receiver on the Raspberry Pi

python main_receiver.py

## Step 3 — Start the sender

On the device sending data (can also be the Pi):

python RpiSend.py

You should see messages like:  Sent to target: {...}  in the terminal.

## What happens if the second server goes offline?

main_receiver.py will automatically ping the target server before each send. If it is unreachable, data is published to AWS IoT and saved to socket_data.csv in the project folder. When the server comes back, forwarding resumes automatically.

# 8. Quick Reference — All Settings

| Variable | Default value | Description |
| --- | --- | --- |
| RPI_HOST | 192.168.1.107 | IP of your Raspberry Pi |
| RPI_PORT | 12345 | Port the Pi listens on |
| TARGET_HOST | 192.168.2.100 | IP of second server |
| TARGET_PORT | 5000 | Port of second server |
| AWS_HOST | …iot.eu-north-1… | Your AWS IoT endpoint URL |
| AWS_TOPIC | test/topic | MQTT topic for AWS IoT |
| ROOT_CA_PATH | certs/AmazonRootCA1.pem | Auto-resolved from project folder |
| CERTIFICATE_PATH | certs/certificate.pem.crt | Auto-resolved from project folder |
| PRIVATE_KEY_PATH | certs/private.pem.key | Auto-resolved from project folder |
| CSV_FILE_PATH | socket_data.csv | Saved beside config.py |